Electric Go-Kart Software — Requirements Specification (Revised)

Version: 1.1

Date: 2026-08-09

Status: Ready for architecture / implementation planning

1. Project Objective

Develop a software platform for a custom electric go-kart that provides:

- Virtual simulation of the complete kart before hardware exists.

- Component-specific modelling using the exact specifications of installed components.

- Performance prediction and tuning before changing physical hardware.

- Real-time vehicle control through ESP32/embedded firmware.

- A driver dashboard for the physical kart.

- Telemetry logging and analysis from the physical kart.

- Hardware-in-the-loop / software-in-the-loop testing.

- Configurable drive modes and speed/power restrictions.

- Fault detection and safety-state management.

- A common configuration that can support both V1 (48 V / ~5 kW peak) and V2 (72 V / ~8–10 kW) without rewriting the software.

The software shall be designed so that the simulation, firmware, and physical kart use the same underlying vehicle configuration and control concepts wherever practical.

Core design principle: A single configuration-driven vehicle model and control architecture. Python simulation, ESP32 firmware, and the dashboard are different interfaces to that shared model. Do not build three independent applications.

2. Scope, Realism, and Phasing

This is an ambitious long-term system. Full vehicle dynamics, sophisticated thermal models, production-grade firmware, HIL, and rich analysis tools represent multi-year effort.

Minimal Viable Product (MVP) target (must be achievable end-to-end before physical kart exists):

- Component / vehicle database + versioning + validation

- Practical physics engine (force balance + efficiency + basic thermal)

- Drive modes + hard safety limits

- Safety state machine (software)

- Simulated real-time dashboard + basic telemetry logging

- Configuration validation against hardware limits

- Clear separation of hardware absolute limits vs configurable limits

All other features are valuable but secondary. The system must deliver a closed loop of configure → simulate → (later) measure → calibrate as early as possible.

3. High-Level System Architecture

The system consists of these major elements:

┌────────────────────────┐
                    │  COMPONENT / VEHICLE   │
                    │  DATABASE + VERSIONING │
                    └───────────┬────────────┘
                                │
                                ▼
┌───────────────────────────────────────────────────────┐
│           SHARED VEHICLE + CONTROL MODEL              │
│                                                       │
│  Configuration  →  Physics  →  Limits  →  Safety      │
│  (data)            (logic)     (logic)    (logic)     │
└───────────────────────┬───────────────────────────────┘
                        │
          ┌─────────────┼─────────────┐
          ▼             ▼             ▼
   ┌─────────────┐ ┌──────────┐ ┌──────────────┐
   │   Python    │ │  ESP32   │ │ Web/Desktop  │
   │ Simulation  │ │ Firmware │ │  Dashboard   │
   └─────────────┘ └────┬─────┘ └──────────────┘
                        │
                        ▼
                 Physical VESC / BMS /
                 sensors / display /
                 contactors

Key architectural rules:

- There is one authoritative Vehicle Configuration (data) and one Vehicle/Control Model (logic).

- Control algorithms (torque request mapping, mode limits, current limiting, safety transitions) shall be designed so the same logic can be unit-tested in Python and faithfully implemented on the ESP32.

- VESC and BMS interfaces shall be abstracted behind driver interfaces. Protocol details must not leak into the rest of the system.

- Safety-critical paths must remain deterministic and independent of UI or telemetry performance.

- All internal calculations use SI units. Display conversion is performed at the presentation layer.

4. Vehicle Configuration System

The user must be able to create a complete digital representation of the kart by entering the actual components installed. The system shall not hard-code V1 specifications.

4.1 Vehicle Parameters

Configurable parameters include:

- Vehicle name and version

- Dry mass, battery mass, driver mass, maximum permitted vehicle mass

- Wheelbase, front track, rear track

- Centre of gravity height and longitudinal position

- Aerodynamic drag coefficient and frontal area

- Rolling resistance coefficient

- Maximum vehicle speed

- Maximum permitted acceleration and deceleration

Multiple named configurations shall be supported (examples: “Scott Kart V1”, “Scott Kart V1 – Wet Tyres”, “Scott Kart V1 – Junior Setup”, “Scott Kart V2”).

4.2 Configuration Versioning

Every vehicle configuration is version-controlled. Historical telemetry remains permanently associated with the exact configuration version, firmware version, and calibration set used.

Example progression:

V1.0 → 48 V / 40 Ah, 12T/52T, 150 A motor current
V1.1 → 48 V / 40 Ah, 14T/52T, 150 A motor current
V2.0 → 72 V / 50 Ah, 14T/56T, 180 A motor current

4.3 Configuration Safety & Validation

All configuration changes are validated before acceptance.

- Hardware absolute limits cannot be raised by user configuration.

- Vehicle configuration limits cannot exceed hardware absolute limits.

- Drive-mode and driver-profile limits cannot exceed vehicle configuration limits.

Rejected changes must clearly state the reason. A configuration change audit log (who/what/when/from-version) shall be maintained.

5. Component Models

5.1 Motor

Electrical and mechanical parameters (nominal/max voltage, continuous/peak current and power, phase resistance, Kv, poles, hall configuration, max RPM, continuous/peak torque, mass, shaft dimensions, cooling method, max temperature).

Preference order for simulation:

- Measured or manufacturer torque vs RPM and efficiency maps (CSV import supported)

- Nameplate continuous/peak values with constant efficiency fallback

Optional: datasheet / PDF attachment, manufacturer notes.

5.2 Motor Controller

Manufacturer, model, firmware version, voltage limits, continuous/peak battery and phase currents, RPM limits, temperature limits, regenerative current limit, efficiency, CAN configuration and message definitions, fault codes.

The simulator and firmware must enforce the minimum of motor, controller, battery, and BMS limits at every step.

5.3 Battery & BMS

Considerably more sophisticated than a simple “48 V 40 Ah” entry.

Electrical: chemistry, nominal/max/min voltage, series/parallel cell counts, capacity (Ah), energy (Wh), cell and pack resistance, continuous/peak/charge/regen current limits.

Thermal: max/min cell temperature, thermal capacity and resistance, cooling method.

BMS: model, current limits, voltage and temperature limits, CAN interface, cell voltage telemetry support.

Simulation shall calculate SOC, pack voltage, current, power, energy consumption, voltage sag, heat generation, remaining energy, and estimated range. Support for SOC-dependent open-circuit voltage and resistance curves is required. Start with a practical equivalent-circuit + thermal-mass model; full electrochemical models are out of scope for early releases.

5.4 Drivetrain

Motor sprocket teeth, axle sprocket teeth, gear ratio, chain type and efficiency, number of stages, gearbox/differential if present, axle efficiency, wheel/tyre diameter and circumference, tyre rolling resistance.

System calculates both kinematic path (motor RPM → vehicle speed) and force path (motor torque → tractive force) and keeps them consistent.

5.5 Tyres

Individual front/rear specifications: manufacturer, model, diameter, width, rim diameter, mass, rolling resistance, dry/wet grip coefficients, maximum rated speed and load, temperature range.

Initial model: simple longitudinal friction + rolling resistance. Later: slip, wheelspin, and lock-up detection.

5.6 Brakes

Brake type, disc diameter/mass, caliper and master-cylinder characteristics, pedal ratio, maximum braking torque, front/rear distribution, regenerative braking strength and maximum regenerative current.

Mechanical braking and regenerative braking are modelled as separate paths. Regenerative braking must respect battery charge-current and motor/controller regen limits.

5.7 Accessory / Low-Voltage System (New)

DC-DC converter, display, lights, contactor coil, and other accessory loads. These affect range and can create brown-out conditions. Basic power budget modelling is required.

5.8 Contactor & Precharge (New)

Explicit modelling and control of the high-voltage connection sequence, including precharge behaviour and contactor state.

6. Component Database

Reusable records for motors, controllers, batteries, BMS units, tyres, wheels, brakes, DC-DC converters, sensors, and other parts.

Each record stores manufacturer, model, part number, datasheet, specifications, source, purchase price, date added, and user notes. Components can be exported/imported.

7. Vehicle Dynamics Simulation

Minimum calculated quantities:

- Vehicle speed, acceleration

- Motor RPM, torque, power

- Battery current, voltage, power, SOC

- Motor, battery, and controller temperatures

- Rolling resistance, aerodynamic drag, gradient effects, mechanical losses

Inputs: throttle, brake, steering, vehicle mass, road gradient, surface condition, drive mode, ambient temperature.

Simple traction / wheelspin limiting (detect excessive motor RPM relative to vehicle speed and temporarily reduce torque request) shall be supported as a high-value early feature.

The simulator shall support accelerated time for long-range runs.

8. Performance Analysis & Virtual Tuning

Supported analyses:

- Acceleration times (0–10 / 20 / 30 / 40 / 50 km/h)

- Theoretical and practical top speed

- Hill-climbing (user-specified gradient and distance)

- Range estimates under different duty cycles and drive modes

Virtual tuning: Change any component or limit parameter and immediately compare resulting performance.

Parameter sweep / optimisation: Automatic combinatorial testing (e.g. sprocket ranges) with user-defined objectives (maximise acceleration subject to top-speed and current constraints). Start with simple combinatorial sweeps; advanced multi-objective optimisation can come later.

Simulation vs real-world comparison: Every logged session is linked to its exact configuration version. The system shall support:

- Summary tables (simulated vs measured)

- Replay of recorded throttle/brake profiles through the simulator with overlaid traces

- Adjustment of model parameters based on real-world data so the simulator improves over time

9. Drive Modes & Driver Profiles

Drive modes are fully configurable (not hard-coded). Each mode may define maximum speed, battery current, motor current, acceleration, throttle response, regenerative braking, power, RPM, and permitted gradient.

Driver profiles (Child, Junior, Owner, Guest, Test, Track, etc.) are configurable and may be protected by PIN, RFID, physical key, or phone authentication.

Limit hierarchy (strictly enforced):

- Hardware absolute limits (immutable by software configuration)

- Vehicle configuration limits

- Drive-mode limits

- Driver-profile limits

10. Safety State Machine & Fault Handling

Explicit states (example sequence):

OFF → BOOT → SELF-TEST → READY → ARMED → DRIVING → FAULT → SAFE SHUTDOWN

Defined behaviour for throttle/brake/wheel-speed faults, CAN timeouts, VESC/BMS faults, over-/under-voltage, over-temperature, overspeed, MCU watchdog, sensor disagreement, contactor faults, and others.

Safety-critical shutdown mechanisms must not depend solely on ESP32 software. Hardware interlocks (contactor control, independent e-stop paths where practical) are required.

Fault injection in the simulator (and later HIL) is mandatory for verification of state transitions.

Power-on self-test coverage and explicit behaviour when critical sensors are missing or out of range are required.

Unit tests for the safety state machine and limit enforcement shall be runnable without hardware.

11. Telemetry, Logging & Analysis

High-frequency logging of timestamp, speed, motor RPM, throttle, brake, torques/currents, voltages, SOC, temperatures, drive mode, fault state, GPS (when available), acceleration, and regenerative power. Sampling rates are configurable.

Session metadata: session ID, date/time, vehicle configuration version, driver, firmware version, hardware configuration, start/end SOC.

Storage: CSV for simple inspection + structured format (SQLite or equivalent) for analysis. Configuration hash/version ID is stored with every session.

Post-drive analysis dashboard provides time-series graphs and calculated metrics (max/average speed, peak/average power, energy consumption, Wh/km, acceleration times, temperatures, regenerated energy, etc.).

Exportable simulation and comparison reports (PDF or HTML) that include key plots and the exact configuration used are required.

12. Dashboard Requirements

Physical driving display prioritises (in order):

- Large speed

- Drive mode

- Battery SOC

- Fault / warning indication

- Power

Complex menus must not be required while the vehicle is moving.

Additional screens (configuration, diagnostics, live telemetry, trip history) are available when stationary and for authorised users.

13. Firmware Requirements (ESP32)

Sensor acquisition, CAN communication, VESC and BMS communication, drive-mode management, throttle and brake processing, speed calculation, safety state machine, fault management, telemetry, data logging, display communication, watchdog, configuration management, firmware version reporting, contactor/precharge control.

Hard upper bounds that cannot be overridden by normal user configuration are mandatory.

Calibration values (throttle, brake, wheel-speed, voltage, current, temperature, steering) are stored separately from the core vehicle configuration and are versioned.

14. VESC Integration

Support reading and writing of motor RPM, currents, voltage, duty cycle, temperatures, fault codes, current/RPM limits, and torque/current commands.

Protocol details are hidden behind a driver interface so the rest of the software is not tied to one controller manufacturer.

15. Hardware-in-the-Loop Capability

Progressive support:

- PC simulator talking the same CAN interface the real hardware will use.

- ESP32 firmware running against simulated vehicle.

- Real throttle → ESP32 → VESC → real motor (simulator no longer required).

16. Calibration & Model Improvement Workflow

Intended engineering loop:

- Enter component specifications

- Build virtual kart

- Simulate performance

- Optimise gearing / current limits

- Build physical kart

- Controlled testing + telemetry

- Compare measured vs predicted

- Calibrate model parameters

- Re-run simulations and retune

This turns the software into an engineering tool rather than only a dashboard.

17. V1 / V2 Compatibility

The software must not assume V1 hardware. The same architecture supports:

- V1: ~48 V, ~40 Ah, ~5 kW peak, ~40 km/h class

- V2: ~72 V, 40–50 Ah, ~8–10 kW peak, ~50–60 km/h class

Selection of vehicle configuration (V1 / V2 / custom) changes data only; application architecture remains unchanged.

18. Future Expansion (Explicitly Out of Scope for Early Releases)

Architecture shall leave clean extension points for: GPS + geofencing, phone app, Bluetooth/Wi-Fi, OTA updates, lap timing, track maps, remote diagnostics, cloud telemetry, multiple vehicles, driver leaderboards, maintenance reminders, advanced battery health / cell-level monitoring, and automatic performance optimisation.

These are not required for the initial useful system.

19. Non-Functional Requirements

- Reliability — Communication failure results in a defined safe state.

- Determinism — Control and safety functions operate predictably and independently of UI performance.

- Modularity — Motor, controller, BMS, and dashboard implementations are replaceable.

- Testability — Core control and safety logic is unit-testable without physical hardware.

- Configuration-driven design — Vehicle specifications are data, not hard-coded constants.

- Traceability — Every telemetry session identifies the exact hardware/software/calibration configuration.

- Offline operation — The kart itself does not depend on an internet connection.

- Recoverability — Failure of display, Wi-Fi, or dashboard application must not prevent basic safety functions.

- Units — Internal calculations use SI units.

20. Suggested Development Priority

- Data model + component/vehicle database + versioning + validation rules + limit hierarchy

- Core physics (force balance + efficiency + basic thermal + simple traction limiting) + real-time simulation loop

- Drive modes + safety state machine + fault injection + unit tests for safety/limits

- Virtual dashboard + basic telemetry logging + session metadata

- ESP32 skeleton (sensors, CAN stubs, state machine, hard limits, watchdog, contactor/precharge)

- VESC + BMS drivers + calibration system

- Real telemetry + measured-vs-predicted comparison + model calibration loop

- Parameter sweeps and basic optimisation tools

- Hardware-in-the-loop and richer dynamics / tyre models

- Advanced features (reports, accessory modelling refinements, etc.)

21. Final Design Mandate

Do not build three independent applications.

Build a shared, configuration-driven vehicle model and control architecture. Python simulation, ESP32 firmware, and the dashboard act as different interfaces to that underlying model wherever practical. Safety-critical behaviour must remain consistent across simulation and real hardware.